Name: _______________________

Practice – 20.2 Ohm’s Law: Resistance and Simple Circuits

1.  The IR drop across a resistor means that there is a change in potential or voltage across the resistor.  Is there any change in current as it passes through a resistor? Explain.

2.  What current flows through the bulb of a 3.00-V flashlight when its hot resistance is 3.60 ?

3.  Calculate the effective resistance of a pocket calculator that has a 1.35-V battery and through which 0.200 mA flows.

4.  What is the effective resistance of a car’s starter motor when 150 A flows through it as the car battery applies 11.0 V to the motor?

5.  How many volts are supplied to operate an indicator light on a DVD player that has a resistance of 140 , given that 25.0 mA passes through it?

6.  A.  Find the voltage drop in an extension cord having a 0.0600- resistance and through which 5.00 A is flowing.

B.  A cheaper cord utilizes thinner wire and has a resistance of 0.300 .  What is the voltage drop in it when 5.00 A flows?

C.  Why is the voltage to whatever appliance is being used reduced by this amount?

7.  A power transmission line is hung from metal towers with glass insulators having a resistance of 1.00×109 .  What current flows through the insulator if the voltage is 200 kV? (Some high-voltage lines are DC.)

Solutions:

1.  No		2.  0.833 A		3.  6.75 x 103 		4. 7.33 x 10-2 

5.  3.50 V	6. A. 0.300 V       B. 1.50 V      C.  voltage is fixed coming out of the wall

7.  2.00 x 10-4 A